## ClarkDietrich® Low-Embodied Carbon (LEC) Metal Lath and Accessories

Section 092400 – Cement Plastering

NOTE:

Turn on hidden text to view important non-printing notes by selecting Show/Hide (¶) in the Paragraph Group, or by using the Ctrl+* keyboard shortcut.

Note to Specifier: The language provided herein is intended to be supplementary to Division 09 Metal Lath Sections. It contains the necessary language for the specification of Low-Embodied Carbon (LEC) Metal Lath and Accessories only. Coordinate with Related Requirements in applicable Division 01 Sections.

- GENERAL

Retain Paragraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections. These may include “Section 013329 – Sustainable Design Reporting,” “Section 013563 – Sustainability Certification Project Requirements,” “Section 013566 – Sustainability Certification Project Procedures,” “Section 018113 – Sustainable Design Requirements” or related Sections.

      - SUMMARY

        - Related Requirements:

          - <Insert Section Number and Title>.

      - ACTION SUBMITTALS

        - Sustainable Design Submittals:

          - Low-Embodied Carbon (LEC) Environmental Product Declaration (EPD): For applicable products specified in Part 2.

            - Product-Specific Type III Environmental Product Declaration (EPD): Third-party verified, in conformance with ISO 14025 and EN 15804.

              - Global Warming Potential (GWP) values in kg CO₂e per declared unit.

- PRODUCTS

      - MATERIALS

        - Low-Embodied Carbon (LEC) Requirements:

          - Global Warming Potential (GWP) Thresholds: For [Metal lath] [Galvanized steel plaster accessories] complying with ASTM A653/A653M.

Maximum 2,228 kg CO₂e/ton cradle-to-gate is a recommended threshold determined by the U.S. General Services Administration.

            - Maximum 2,228 kg CO₂e/ton cradle-to-gate.

      - METAL LATH

        - Expanded-Metal Lath: ASTM C847, cold-formed carbon-steel sheet with ASTM A653/A653M, G60 (Z180), hot-dip galvanized-zinc coating.

Retain "Basis-of-Design Product" Paragraph below to require a specific product or a comparable product from manufacturer listed.

          - Basis-of-Design Product: Subject to compliance with requirements, provide ClarkDietrich; Diamond Mesh Lath or comparable product by a current member of the SFIA.

          - Diamond-Mesh Lath: [Flat] [Self-furring], [2.5 lb/sq. yd. (1.4 kg/sq. m)] [3.4 lb/sq. yd. (1.8 kg/sq. m)].

3/8-inch (9.5-mm) rib lath is often used where its rigidity is necessary for long spans.

          - High Rib Lath: 3/8-inch (9.5-mm) rib lath with herringbone mesh pattern; 3.4 lb/sq. yd. (1.8 kg/sq. m).

      - ACCESSORIES

        - General: Comply with ASTM C1861, and coordinate depth of trim and accessories with thicknesses and number of plaster coats required.

        - Metal Accessories:

Retain "Basis-of-Design Product" Paragraph below to require a specific product or a comparable product from manufacturer listed.

          - Basis-of-Design Product: Subject to compliance with requirements, provide weep and sill screeds, corners, casing beads, control joints and expansion joints by ClarkDietrich or comparable products by a current member of the SFIA.