This Shortform Specification is intended to be used only as a “Guide” for adding product information to existing full-length Specification Sections. The user accepts responsibility for all Project Specifications. ClarkDietrich bears no responsibility for errors or omissions in any portions of the Project.

ClarkDietrich® Interior Finishing

SECTION 092900

GYPSUM BOARD

NOTE:

Turn on hidden text to view important non-printing notes by selecting Show/Hide (¶) in the Paragraph Group, or by using the Ctrl+* keyboard shortcut.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Trim accessories.

          - Joint treatment materials.

Retain "Related Requirements" Paragraph below to cross-reference requirements Contractor may expect to find in this Section but are specified elsewhere. These may include but are not limited to: Section 061600 “Sheathing,” Section 092116 “Gypsum Board Assemblies,” Section 092116.23 “Gypsum Board Shaft Wall Assemblies,” Section 092216 “Non-Structural Metal Framing,” Section 092613 “Gypsum Veneer Plastering,” Section 093013 “Ceramic Tiling,” or related Sections.

        - Related Requirements:

          - <Insert Section Number and Title>.

- PRODUCTS

      - TRIM ACCESSORIES

        - Interior Trim: ASTM C1047.

          - Material: [Galvanized steel sheet, rolled zinc, vinyl, PVC-based composite, or paper-faced galvanized-steel sheet] [Galvanized steel sheet or rolled zinc] [Vinyl] [PVC-based composite] [Paper-faced galvanized-steel sheet].

          - Shapes:

            - Cornerbead.

              - Product: ClarkDietrich; [103 Deluxe Corner Bead] [Paper-Faced Corner Bead] [Quicksilver Corner Bead] [Strait-Flex Original] [Vinyl Corner Bead] [West Coast Corner Bead] or comparable product.

            - Bullnose bead.

              - Product: ClarkDietrich; [Bullnose Corner Bead] [Paper-Faced Bullnose Tape-On Cornerbead] [Vinyl Bullnose Corner Bead] or comparable product.

            - LC-Bead: J- or U-shaped; joint compound is applied to exposed long flange.

              - Product: ClarkDietrich; [Metal U-Trim #200-A] [Muddable J-Bead] [Rip-Bead Muddable J-Bead] or comparable product.

            - L-Bead: L-shaped; joint compound is applied to exposed long flange.

              - Product: ClarkDietrich; [Metal L-Trim #200-B] [Rip-Bead Flat L-Bead] [Rip-Bead L-Bead] [Strait-Flex L-Bead] [Vinyl L-Bead] or comparable product.

            - U-Bead: J-shaped; does not require joint compound.

              - Product: ClarkDietrich; [Metal J-Trim M400 Series] [Vinyl J-Bead] or comparable product.

            - Control Joint.

              - Product: ClarkDietrich; [093 Control Joint] [093 Vinyl Control Joint] or comparable product.

            - Curved-Edge Cornerbead: With notched flanges that conform to curves and arches.

              - Product: ClarkDietrich; [PRO-90 Archway] [PRO-Bull 3/4" Archway] [Strait-Flex Arch-Flex] [Strait-Flex Arch Stick] or comparable product.

          - Flexible Corner Types:

            - Wall Corner Trim (Stick Corner Beads):

              - Product: ClarkDietrich; [Strait-Flex OS-400] [Strait-Flex Big-Stick] [Strait-Flex Wet-Stick] or comparable product.

            - Inside Corner:

              - Product: ClarkDietrich; [Strait-Flex Original] [Strait-Flex IS-300] [Strait-Flex Pro-Flex 300] or comparable product.

            - Edge Trim:

              - Product: ClarkDietrich; Strait-Flex Edge-Tape or comparable product.

            - Vaults/Soffits:

              - ClarkDietrich; [Strait-Flex Gold] [Strait-Flex Pro-Flex 300] or comparable product.

      - JOINT TREATMENT MATERIALS

        - General: Comply with ASTM C475/C475M.

        - Joint Tape:

          - Interior Gypsum Board: Paper-based; general use.

            - Product:  ClarkDietrich; Strait-Flex Butt-Tape or comparable product.

          - Tile Backing Panels: Composite; for use with fiber cement board, green board, mold-resistant board, or water-resistant board in wet areas.

            - Product: ClarkDietrich; Strait-Flex Tile-Tape or comparable product.

- EXECUTION

      - INSTALLATION OF TRIM ACCESSORIES

        - General: Install in accordance with ASTM C840 and manufacturer’s written instructions.

        - Control Joints: Install [at locations indicated on Drawings] [in accordance with ASTM C840 and specific locations approved by Architect].

        - Interior Trim: Install in locations as follows:

Cornerbead: At outside corners [unless otherwise indicated] <Insert requirements>.

Bullnose Bead: [At outside corners] [As indicated on Drawings] <Insert requirements>.

LC-Bead: [At exposed panel edges] [As indicated on Drawings] <Insert requirements>.

L-Bead: [As indicated on Drawings] <Insert requirements>.

U-Bead: [At exposed panel edges] [As indicated on Drawings] <Insert requirements>.

Curved-Edge Cornerbead: [At curved openings] [As indicated on Drawings] <Insert requirements>.

      - APPLYING JOINT TREATMENT MATERIALS

        - General: Install in accordance with ASTM C840 and manufacturer’s written instructions.

        - Apply joint tape at gypsum board and tile-backing panel joint locations unless otherwise indicated.